New FSM Method Notes:

• Special J Reduction: because we listed the ‘1’ in the excitation data for the J input based solely on

a set transition in the Y1 Y1+ state variable, we know the current state of Y1 is a ‘0’. If the

current state of Y1 is a ‘0’, then the complement of Y1 (Y1 ), must be a ‘1’. Because we know in

this case that Y1 must be in the ‘1’ state, the equation  Y2 Y1  J1 ⋅=  can be written as  Y2 1  J1 ⋅=

which further reduces to simply J1 = Y2.

• Special K Reduction: because we listed the ‘1’ in the excitation data for the K input based on a

clear transition on the Y1 Y1+ state variable, we know that the current state of Y1 is a ‘1’. If the

current state of Y1 is a ‘1’, then the equation Y2Y1K1 ⋅=  can be written as Y2 1  K1 ⋅=  which

further reduces to K1 =Y2.

D excitation equation: Set and HOLD1 transitions

T excitation equation: Set and clear transitions

J excitation equation: Set transitions

K excitation equation: Clear Transitions

Simplify one hot equations to have no negated date values (Y).